Altitude and GPSs

# Introduction

Sopot Bulgaria 2008. The Paragliding World Cup task 3 saw a melt-down as scorers found that many pilots had infringed airspace and were given zero scores. Some of the penalised pilots were adamant that they were innocent and in some cases were subsequently proved to be correct. One of the problems was misinterpretation of the information displayed on the flight instruments and a misunderstanding of what was being recorded in the track log.

This article aims to highlight the differences between barometric altitude and geometric height (height above mean sea level) and explain the quirks of a variety of flight instruments. Once these points are understood then height restrictions may be defined clearly and without ambiguity and pilots will be able to read and interpret their instruments correctly. The article concludes with some suggestions on how organisers can best declare height limits in competitions.

For an introduction on GPSs and their use in competitions I have previously written an article on the flylaragne website: http://flylaragne.com/opens/gps.htm

# GPS and geometric height

In 1973 the US decided to develop what we know today as the GPS system. This required 24 satellites for full functionality, the system was officially fully active in 1995. The scale diagram to the right illustrates the orbits of the satellites; each one is positioned 20200km above the surface of the earth and orbits the earth in 11 hours and 58 minutes (out of interest geostationary satellites for television are twice the distance away). They are each travelling at 3900m/s, so fast that Einstein’s theories of relativity impact how the time of the clocks on board appear different to those on earth (not that this is important).

The GPS receiver that we carry needs to receive the signal from a minimum of 3 satellites to determine its 2 dimensional location or a minimum of 4 satellites to determine its 3 dimensional location. Whilst we are flying it is quite normal to ‘see’ more satellites than this, resulting in a more accurate location. Each of the satellites has an on board atomic clock and transmits its position and the time to Earth; the fly-time of this transmission from the satellite to the Earth is about 70milliseconds. Your GPS unit takes the time from each satellite and triangulates its location in space by calculating the difference in fly-time between satellites.

So, your GPS unit knows where the satellites are at any given time; it knows where it is (by triangulation) and it needs to tell you where you are relative to its model (which is selectable) of the Earths surface.

Your GPS unit has multiple models (datums) of the earth in its memory. The most commonly recognised global model is the World Geodetic System established in 1984 (WGS84). There are other datums that may be more accurate for individual countries but in the glider world we tend to use WGS84.

WGS84 defines the earth as an ellipsoid; a squashed ball. This ellipsoid is a pretty good approximation to the mean sea level around the planet, but is recognised as having errors of between -100m and 70m with respect to the geoid, dependent upon where you are on the globe.

The geoid is the best definition of mean sea level (msl). The geoid is defined as ‘The particular equipotential surface that coincides with mean sea level and that may be imagined to extend through the continents. This surface is everywhere perpendicular to the force of gravity.’ (www.nps.gov)

Picture the geoid as the ellipsoid with lumps on. The figure below shows a graphic representation of where the errors lie. In Europe the ellipsoid is about 50m above the geoid. In southern India it is nearly 100m below. Significantly, even though we select the WGS84 datum (ellipsoid), some GPSs add the geoid correction to give geometric height and some don’t. A calculator for the ellipsoid to geoid height for any point on the globe can be found here: http://sps.unavco.org/geoid

## GPS Errors

| There are additional errors involved in calculating the GPS position. The errors shown in the table below, exist in the X/Y axes (latitude and longitude); the vertical Z axis has around 3 times these errors due to the satellites’ positioning above the globe. Total accuracy is better than ±15m in the horizontal, and about ±45m in the vertical axis. If the satellite reception is poor these errors will increase, but whilst flying the reception is usually good. GPS horizontal errors |  |
| --- | --- |
| Ionospheric effects | ± 5m |
| Shifts in the satellite orbits | ± 2.5m |
| Clock errors of the satellites' clocks | ± 2m |
| Multipath effect | ± 1m |
| Tropospheric effects | ± 0.5m |
| Calculation and rounding errors | ± 1m |
| Total | ±15m |

Different Datums and use of the geoid or ellipsoid will give variable locations: It is essential that everyone in a competition, for example, uses the same datum.

There was a case in 2008 when the scoring software used distances around the ellipsoid and one GPS instrument used the FAI sphere (a sphere of radius 6371km). The scoring program considered the pilot was outside of the 3km cylinder by 3m when his instrument declared him inside by 4.5m (http://www.flytec.ch/download/Faq/Distance calculations.pdf). There is a recommendation by the Paragliding World Cup and in the CIVL FS scoring program that 0.5% tolerance be set in the distance calculations of the scoring software to eliminate this problem.

When the GPS displays the height information, it is normally averaged over a few seconds. This leads to an accurate reading while stationary, but introduces a delay in the reading whilst climbing or sinking.

Satellite Based Augmentation Systems are partially operational and are being developed for use in the commercial aviation sector. WAAS, EGNOS and MSAS are satellite based differential GPS systems that improve the accuracy of the GPS. The three systems are based in the US, Europe and Asia respectively and are all compatible with each other. When WAAS was enabled, my Garmin 76S worked in Australia but does not yet in Spain.

For more information on the GPS system see this website: http://www.kowoma.de/en/gps/index.htm

# Barometric Altitude

The aviation industry has always used barometric pressure to measure altitude. Altimeters measure pressure and are calibrated to indicate that pressure as an altitude in feet or meters. The calibration assumes that the pressure drops at a standard rate as altitude is gained. We are interested in the altitude above ground level (agl) or above mean sea level (amsl) or with respect to the standard reference of 1013.25hPa (Pressure Altitude, PA). There is no evidence to suggest the industry will move to GPS altitude in the near future.

On any given day the pressure at mean sea level (msl) will vary due to meteorological effects, and the pressure will reduce as altitude increases. Pressure is measured in hectoPascals (hPa) or millibars (mb), which are essentially the same. From sea level the pressure decreases with altitude at 27ft/hPa, rising to 30ft/hPa at 4000ft (1220m) and 40ft/hPa at 15000ft (4570m): It is normal to assume 30ft/hPa where we fly.

The International Civil Aviation Organization (ICAO) defines a model formula for height, pressure and temperature. Flytec has derived the following height and pressure equations from the ICAO formula: (documented at http://www.flytec.ch/download/Faq/Hoehenkorrektur Englisch.pdf).

|  | ## Height equation |  | ## Pressure equation |
| --- | --- | --- | --- |
|  | h=273.15+T00.0065.1−PP015.255 |  | P=P0.1−0.0065h273.15+T05.255 |
|  | h = height (m) P = Actual pressure at height h (hPa) |  | P0 = Reference Pressure (hPa) T0 = Standard Temperature (15°C) |
|  | The ICAO gives the standard pressure to be 1013.25hPa and the standard temperature is 15°C at sea level. |  |  |

For any given pressure (P) we can calculate the height (h) above the reference pressure (P0), or vice versa with a given height (P0 can be the standard pressure or any QNH). Altimeters are designed and calibrated to meet this formula. It must be noted that the altitude indicated by an altimeter is temperature compensated. For any given pressure the indicated altitude will be the same no matter what the temperature. This can give rise to a significant difference between the GPS height and the displayed barometric altitudes.

## Airspace

To remain safe and legal we fly according to Visual Flight Rules, and we have to avoid controlled airspace. Normally for a given area there is a transition layer, above which airspace will be defined at a certain flight level. Below the transition layer airspace is defined as altitude amsl.

For example (figures from a Cathay Pacific pilot):

In Australia the transition layer starts at ALT10000 ft amsl and finishes at FL110.

In New Zealand and most of Asia the transition layer starts at ALT11000 ft amsl and finishes at FL130.

In the USA and Canada the transition layer starts at ALT18000 ft amsl and finishes at FL180.

In Europe, Middle East, Russia and China the transition layer varies, and can be as low as 3000ft agl.

When flying below the transition layer, altimeters should be set to the local QNH (Query Nautical Height); the pressure at msl for that area. The altimeter will indicate altitude amsl. Restricted airspace below the transition layer will be expressed as altitude amsl.

When flying above the transition layer, altimeters should be set to the standard pressure of 1013.25hPa (or QNE; Query Nautical Elevation). The altimeter will indicate Pressure Altitude (PA). To avoid confusion between Pressure Altitude and Altitude, PA generally has the last two zeroes removed and is then known as flight level: PA10000 is more commonly known as FL100. Restricted airspace above the transition layer will be expressed as a Flight Level.

It is essential to know all of the airspace restrictions in the area you are flying. It is possible that you will have a combination of Flight Level and Altitude restrictions during a flight.

# Differences between Barometric and GPS height

Assuming that the GPS displays the geometric height above the geoid and that the signal reception is good then the GPS will display the height amsl ±45m. For any given geometric height, the indicated altitude of the altimeter (the one that defines airspace) will vary depending on the conditions of the day.

The problem with the altimeter’s indicated altitude is that it is calibrated assuming a standard day: That with increasing altitude the pressure drops at a standard rate. This assumption is not very accurate for two major reasons.

- If the temperature at sea level is anything other than 15°C then the rate of drop will not be standard.

- The atmosphere usually has hot and cold layering making the rate of change of pressure non-uniform.

It is possible to take the height equation above and substitute T0 with the day’s temperature at sea level (the temperature gradient used in the formula is -6.5 degrees per 1000m; use this to calculate the temperature at sea level).

The diagram below illustrates how Flight levels change their height depending on the day. FL100 is used as an example and is always at 697hPa.

Day 1 is with QNH=standard pressure and the temperature is standard 15°C. In this case FL100 is at an altitude of 10000ft amsl because QNH = QNE = 1013.25hPa. Also the GPS will measure a height of 10000ft above the geoid and everyone is happy.

Day 2 has a high pressure system giving a QNH of 1030hPa but is still a cool 15°C at sea level. The green altimeter, set to QNE, will still indicate FL100 as 10000ft. Using the height equation, we can determine that at a pressure of 1030hPa, the yellow altimeter will read 454ft higher than the green altimeter at any altitude. Therefore the altitude indicated for FL100 will be 10454ft amsl. The geometric height, as displayed by a GPS, will be 10420ft, at FL100, which can also be calculated using the height equation. This geometric height is quite close to the 10454ft indicated by the altimeter set to QNH. For illustration purposes the geometric height of 10000ft, as indicated by the yellow altimeter, equates to an altitude amsl of 10031ft at a pressure of 708hPa.

Day 3 has a high pressure system giving a QNH of 1030hPa but is now a hot 40°C at sea level. The green and yellow altimeters will indicate the same as day 2 because they are temperature compensated. Using the height equation with T0=40°C, 697hPa (FL100) equates to a geometric (GPS) height of 11325ft. It has moved up. This geometric height is NOT close to the 10454ft indicated by the altimeter set to QNH, it is 871ft (8%) higher. For illustration purposes the geometric height of 10000ft equates to 9230ft amsl as indicated by the yellow altimeter at a pressure of 731hPa.

From this we see that a change in QNH pressure alone does not cause a large difference between GPS height and indicated altimeter altitude.

But, a change in the msl temperature does create a difference between GPS height and indicated altimeter altitude. The error introduced by the temperature change is 0.347% per 1°C above 15°C and -0.347% per 1°C below 15°C. The reason for this is that the ICAO formula assumes that the density of the air at a particular altitude is standard but in reality this density changes with temperature.

Does this theory work in reality? Below are three flights made in Australia in2009 using a Flytec6030 (and a paraglider). The first was 4 hours, the second 3 hours and the third 6 hours. The day temperature was about the same 31°C in each. The GPS height (y axis) is plotted against the Pressure Altitude (x axis) in each and the trend line (best fit line) automatically inserted by the Excel graphing software. The trend line is a straight line with a general formula of y=mx+c. c is a constant offset in meters with the value of the height difference QNH – QNE on the day. m gives the steepness of the GPS height to Pressure Altitude line. If m is 1 then the GPS height increases at the same rate as the Pressure Altitude; we would like this in an ideal world but we are expecting m to increase as the temperature increases indicating the percentage error.

In flight 1, c needs to be forced to a value of -40m to account for the pressure of 1008hPa whereas for flights 2 and 3 c is zero because the day pressure happened to be 1013hPa.

We are more interested in the m value. In flight 1, m is 1.067, meaning that the GPS altitude is 6.7% higher than the Pressure altitude. Flight 2 the GPS altitude is 7.8% higher than the Pressure Altitude and flight 3 5.5%. According to the theory above, the GPS altitude should be 5.2% higher than PA for a 15°C rise above standard, so in these examples reality is actually worse than the prediction. One explanation of this is that the atmosphere layering could have an influence (reason 2 above). Almost certainly the atmosphere was not standard.

Notice that the graphs have a distribution around the trend line. This could be due to temperature and QNH changes throughout the duration of the flight. Also, due to the delay in the GPS height calculation, strong climbs will plot points slightly to the right and strong sink slightly to the left of the correct value, giving the line a marked thickness.

These flights show conclusively that there is a percentage difference between GPS height and barometric altitude and that the difference can vary, depending on the day and can be worse than the theoretical prediction. It would be good to repeat this using the same instrument on a 15°C day to see that the difference was closer to zero.

All of the calculations, information and graphs for the above are available here: http://flylaragne.com/opens/gps_spreadsheet.zip

# Instrument displays and recording

Detailed here are some of the instruments we use, what they display and what they record; this is not always the same thing.

GPS only instruments: Some Garmins without pressure sensors and MLRs have no barometric capability at all. These instruments can only display and record GPS height. The Garmins always reference the height above the geoid (not the ellipsoid).

Garmin GPSs with a pressure sensor are some of the most common units used. The 76S and 76CS(x) and the 60CS(x) are very popular first instruments and are good for cross country flying particularly when loaded with local maps. The units can display ‘GPS elevation’ which is only on the satellites page, and ‘elevation’ (barometric) is displayed on the elevation page and is a selectable field on all of the other pages. It is this ‘elevation’ that is recorded in the track log. There are two methods of calibrating the barometric altimeter: Auto and Manual.

- In auto calibration mode, the altimeter is continually, but slowly, updated with the GPS height. When a Garmin 76S was set 200m too high it took 90 minutes to autocalibrate. In effect the ‘elevation’ displayed is a combination of GPS height and barometric altitude. It is the GPS height with the more sensitive changes of barometric pressure superimposed. This combination of GPS and Altimeter height makes the information recorded ambiguous. It is probably best to leave autocalibration off and calibrate the altimeter manually.

- In manual calibration mode the pilot must set the height or pressure (QNH or QNE) for the day. After this, the ‘elevation’ displayed, and recorded in the track log is purely based on the altimeter. The altimeter can be calibrated at any time.

It is significant that the status of the altimeter and its calibration is not recorded in the track log.

Flytec (6030, 6020, 5030 and 5020) and Bräuniger (Compeo and Competino series) are comprehensive flight computers that are combined GPS and altimeters. They make available to the track log both GPS height referenced to the geoid, and barometric altitude referenced to standard pressure 1013.25hPa. The Instrument has 2 barometric altimeters (alti1 and alti2) which can be independently set and user fields can be set to display GPS height or barometric altitude. The barometric altitude displayed can be with respect to the user set QNH or QNE. Bear in mind that if the unit is set with QNH, then the track log still contains the information with respect to 1013.25hPa (QNE). It is good that all of the instruments record altitude with respect to the same reference, but is not necessarily the actual amsl for the day. It is in fact the Pressure Altitude and is the information required for avoiding airspace flight above the transition layer. Final glide calculations are made using alti1 so this is best set to QNH.

The Aircotec XC Trainer and Top Navigator are also flight computers that display and record barometric pressure with respect to the QNH calibration. In both instruments the altimeter can be manually calibrated to an altitude or QNH. Once the track log starts to record they cannot be re-calibrated without turning the instrument off. The XC Trainer will normally automatically calibrate the altimeter when it is first switched on after acquiring the GPS position. It will calibrate to the acquired GPS height above the ellipsoid. This calibration can be manually over-ridden before the track log starts to record. GPS height is not recorded in the track log.

Digifly manufactures the Leonardo Pro flight computer. It records both GPS height and Barometric Altitude with respect to the QNH setting. The GPS height is referenced to the geoid.

The CPilot pro started life as software for Palm PDAs but now the hardware is being developed and will soon be launched. It is similar to the Flytecs in that it records both GPS height and Pressure Altitude (referenced to 1013.25hPa). The GPS height is referenced to the ellipsoid.

The Renschler CoMo OLC flight computer records the barometric altitude with respect to the QNH calibration. Actually alti2 is recorded and alti2 is also used for the final glide calculator. Any GPS height information is with respect to the ellipsoid.

| Instrument | WGS84 height wrt | GPS Track | Barometric Track |
| --- | --- | --- | --- |
| Garmin with no pressure sensor | Geoid | Yes | No |
| MLR | ??? | Yes | No |
| Garmin with pressure sensor | Geoid | No | Yes, QNH1 |
| Flytec 6030/ Brauniger Compeo+ | Geoid | Yes | Yes, QNE |
| Aircotec XCTrainer / Top Nav | Ellipsoid | No | Yes, QNH |
| Digifly Leonardo | Geoid | Yes | Yes, QNH |
| CPilot pro | Ellipsoid | Yes | Yes, QNE |
| Renschler CoMo | Ellipsoid | No | Yes, QNH |

1: QNH if the auto calibration is turned off

# Competition problems

Altitude restrictions in force during a competition create headaches for everyone concerned. Pilots wishing to do well will fly as close to the limits as possible and sometimes break them; deliberately or accidentally. The competition organisation has to make sure that the rules are not broken and unfortunately has to award penalties to hammer the point home that infringing airspace is dangerous, as well as unfair. Section 7b (2008) of the FAI sporting code states:

The penalty for exceeding altitude limits that have either been specified at task briefing or published in airspace charts shall be a warning for the first infringement of less than 100m by a pilot. For infringements greater than this or for subsequent infringements the pilot shall score zero for the day.

The scorer of the competition needs to know the quirks of all the instruments presented to him. When the instrument and scoring program don’t agree on something it is a truly difficult situation to resolve.

How should we deal with height restrictions? Here are some suggestions:

## On days when the restrictions are altitude amsl then all of the instruments should be set to the lowest predicted QNH for the day for the region of the task (Garmins must have altimeter calibration set to manual), this should be announced on the task board. MLRs (and Garmins with no pressure sensors) will only record GPS height and the onus is on these pilots to stay clear of airspace, knowing that the hotter the day the more they will over-read. As long as there is one instrument in the competition that gives a GPS / PA dual track, the scorer can extract the GPS height of the airspace restriction and verify all is ok for the GPS only instruments. The scorer will need to take the 1013.25hPa derived traces (Flytecs/Bräunigers) and add the relevant 1013.25 - QNH * 27ft/hPa to get the actual flight altitude. The scoring software could be made to automatically correct these traces using the QNH previously announced. For the instruments that record pressure relative to QNH, it may be necessary to stipulate that the track log is unbroken throughout the flight. It is feasible that the instrument can be switched off in flight and the altimeter calibration changed in order to cheat, though why anyone would want to cheat is beyond me.

## On days when the height restriction is FL, then there are two options:

Option 1: All of the instruments should be set to 1013.25hPa, QNE. The barometric data recorded by all of the instruments will have a common 1013.25hPa reference making life easier for the scorer. The instruments with only one altimeter display (eg the Aircotecs) will display Pressure Altitude (good for avoiding airspace) but there will be a problem with the final glide calculator because zero feet will not necessarily be at sea level.

Option 2: All of the instruments should be set to the lowest predicted QNH for the day as for the altitude amsl case above. The Aircotecs in this case will not show Pressure Altitude and will need to fly remembering the QNE to QNH height difference for the day to calculate the altitude of the FL restriction. Those pilots with Alti2 or a second altimeter would set that to QNE to be sure of the altitude of the FL restriction. The pilots with GPS only instruments will have the same problems as in 6.1 above. The scorer will have to take the QNH derived traces and subtract the QNH - QNE height for the day if there are airspace queries.

The second option is probably the more acceptable one. It means the same procedures are followed whether the height restriction is amsl or FL and also in the rare case that there are both. Final glide calculators will operate correctly. It is quite normal to calibrate instruments (alti1) to QNH rather than QNE. It is then simple to calibrate alti2 to QNE.

## GPS height could be used for both of the cases above. The competition organisers would have to declare the height restrictions with a good enough margin built-in that they don’t get it wrong and have a competition full of pilots infringing airspace (it could happen).

The problem instruments would be the ones that do not record GPS height: the majority of the Garmins with pressure sensors, Aircotecs and Renschlers.

# Conclusions

- Barometric altitude is the standard used by the aviation industry.

- GPSs display altitude as a distance whereas Altimeters display altitude based on pressure measurements. The two are fundamentally different and you would expect them to differ. They more or less indicate the same at 15°C, but the gap widens as the atmosphere becomes non-standard. They differ more as the temperature moves away from the standard 15°C.

- GPSs give height with respect to either the geoid or the ellipsoid; these can differ by up to 100m. There can be ± 45m of errors on top of this with good signal reception, more with bad reception.

- The information available in the instruments’ track log varies between manufacturers.

- Not only must the pilot know what is being recorded in the track log, he must be able to see the values whilst flying.

- Scorers of competitions need to know what information is being recorded in the track log.

- Scorers of competitions need to know whether GPS height or barometric altitude is being specified in the competition task.

- All of the flight computers are firmware driven. It should be possible to change the type of data logged if required.

- I believe we should be using Barometric pressure in competitions (and cross country flying) to avoid airspace, the same as all other aircraft. I do not like the idea of using GPS height for avoiding altitude restrictions. All of the altitude restrictions are based on barometric pressure and it seems wrong to use something different. We are trying to avoid aircraft that are all using barometric instruments, can we guarantee that by using GPS height we will stay clear and safe?

- I think the 100m leeway for the first airspace infringement in a competition should be much less. It is the responsibility of the pilot to know what his instrument is displaying and recording. If the instrument is displaying the pilot in airspace then he should be penalised. The pilot needs to anticipate areas of strong lift and completely avoid airspace.

- Track logs should be unbroken from take off to landing. A break in the track log could indicate that the instrument has been switched off and re-calibrated.

- None of this is a problem if there are no altitude restrictions in the area of the competition.

Thanks to Joerg Ewald, Garmin, Aircotec, Renschler, Digifly and Cpilot for helping me compile this information. A special thank you to Erich Lerch from Flytec for his enthusiastic support.